#### JP Series 4 Specifications

| Model |  | JP-104 Series | JP-204 Series | JP-504 Series | JP-1004 Series |
| --- | --- | --- | --- | --- | --- |
| Pressing Capacity | Maximum | 1,000N(100Kg) | 2,000N(200Kg) | 5,000N(500Kg) | 10,000N(1,000Kg) |
|  | Allowable Range | 100 ~ 1,000N | 200 ~ 2,000N | 500 ~ 5,000N | 1000 ~ 10,000N |
|  | Increments | 1N | 1N | 1N | 1N |
|  | Minimum Sensing Capacity*1 | 0.2% of maximum pressure capacity |  |  |  |
| Ram Stroke | Maximum | 80mm | 80mm | 100mm | 100mm |
|  | Allowable Range | 0 ~ 80mm | 0 ~ 80mm | 0 ~ 100mm | 0 ~ 100mm |
|  | Increments | 0.001mm | 0.001mm | 0.001mm | 0.001mm |
| Ram Speed | Approaching | 166mm/s | 166mm/s | 166mm/s | 166mm/s |
|  | Pressing | 0.01 ~ 35mm/s | 0.01 ~ 35mm/s | 0.01 ~ 35mm/s | - ~ 35mm/s |
|  | Increments | 0.01mm/s | 0.01mm/s | 0.01mm/s | 0.01mm/s |
| Holding Time | Maximum*2 | 999sec | 999sec | 999sec | 999sec |
|  | Allowable Range | 0 ~ 999sec | 0 ~ 999sec | 0 ~ 999sec | 0 ~ 999sec |
|  | Increments | 0.1sec | 0.1sec | 0.1sec | 0.1sec |
| Repeatability |  | ±0.005/mm | ±0.005/mm | ±0.005/mm | ±0.005/mm |
| Load Sensing Range |  | 100 ~ 1,000N | 200 ~ 2,000N | 500 ~ 5,000N | 1,000 ~ 10,000N |
| Sensor Functions | Const. Speed/Set Stop Position | (1) Load, position, distance control at end pressing |  |  |  |
|  | Const. Speed/Set Stop Load | (2) Load control in sensing position or distance range |  |  |  |
|  | Const. Speed/Set Dist. Mode | (3) Load control (peak, top, bottom, valley, peak to peak differential) in sensor position or distance range |  |  |  |
|  | Multiple Section Pressing | The multiple-section pressing mode in combination with the above three modes |  |  |  |
|  | Cont. Load/Set Time Stop | These items will be equipped with from Version 5.5. |  |  |  |
|  | Cont. Load/Set Position Stop |  |  |  |  |
|  | Cont. Load/Set Distance Stop |  |  |  |  |
| Screen Display | Unit of Measure Pressure | N Kgf Lbf mm in |  |  |  |
|  | Display Language | English, Japanese, Spanish, Italian, French, German |  |  |  |
| Main Program Providing System |  | Installed before shipment. To upgrade the program download a new one from PC to the internal flush ROM using the COM1 |  |  |  |
| External Interface*3, *4 |  | RS-232C 1ch (9-pin male connector. PC can be connected to control the press using optional software.) |  |  |  |
| External I/O Terminal Number |  | Inputs 17, Outputs 16 (Optional inputs 24, outputs 24 in total) |  |  |  |
| Driving and Control Method |  | AC servomotor driven by 32-bit CPU |  |  |  |
| Power Supply | Tree-Phase | 180-250V |  |  |  |
|  | Single-Phase | 180-250V |  |  |  |
| Power Capacity |  | ## 300VA | ### 300VA | ## 850VA | ### 850VA |
| Maximum Jig Weight Hanging on the Ram Tip |  | 10N | 20N | 50N | 100N |
| Ambient Temperature |  | 0 ~ 40ºC |  |  |  |
| Relative Humidity |  | 20 ~ 90% Non-condensing |  |  |  |
| Flotage Dust |  | Nil |  |  |  |
| Corrosive Gas |  | Nil |  |  |  |

*1 Load display accuracy is ± 1% and guaranteed within the above ranges, over 10% of the maximum pressure capacity (Check the load value and calibration amount periodically.)

*2 The holding time becomes shorter as the maximum pressure increases due to motor capacity.

*3 The encoder output interface is available optionally. It is useful when position information is required to send to an external device in real time.

*4 The load cell output interface is available optionally. It is useful when load information is required to send to an external device in real time.

*5 You can choose an installation method of the unit type from side surface fixing (standard) or flange vertical fixing (optional.)

*6 The I/O connector does not include cables.

| Stand-Alone Type: | Body (with the area sensor short connector and I/O connector *6), Operation Manuals (supplied in CD-ROM) |
| --- | --- |
| Head Type: | Body (with the area sensor short connector and I/O connector *6), Switch Box (with a 2mm-cable), I/O Cable (3m). |
| Unit Type: | Body, Control Box (with the area sensor short connector and I/O connector *6), Teaching Pendant (with a 2m-cable) 2 Connection Cables between the body and control box (3m), Operation Manuals (supplied in CD-ROM), Teach Pendant Short Connector |
| Optional Devices: | Switch Box for the unit type (with 2m-cable) PC software for result data collecting system and teaching data control systems compiled with Windows2000/XP (Windows is a registered trademark of Microsoft Corporation) |
| ##### 31 Fairfield Place, West Caldwell, NJ 07006 www.fancort.com/airpress |  |